1)If you are going to use API for formulations, for what type of formulations. Tablets, capsules, injectables etc, first it is required to find out QTPP ie Quality Target Product Profile as per QbD, and what are different types of unit processes ie chemical processes and unit operations Like crystallization, drying, humidification etc  are to be carried out and what precautions we need to take care of it ie.risk analysis ie, ICH Q9.based on that inputs and outputs will be decided. Once input and out outs are decided one can use DOE as per ICH8. Here you may have to list out what solvents, catalyst, no of recatants,temperature etc will be participating in the reaction. Similarly what outputs are to be inbuilt in API based on QTPP.Then DOE can be applied. Example is given next para.

2) If you are going to develop only API as it is then also specifications must be known like IP,USP etc. then also it is required to finalise unit processes like hydroxylation, sulfonation, coupling reaction etc. unit operations .for edach of this unit process or unit operation, we need to list out inputs and outputs .then you can apply DOE.

3) we have prepared a template for capturing the data required to plan the Reactions which is as per following.

Format for planning the experiments through DOE

Template for planning DOE. An example.

- Nature of the reaction:

a). Product					         	IBR2

b). Type of the reaction.			Coupling Reaction

c). Exothermic or endothermic reaction.		normal

d). Reversible or irreversible reaction.		Irreversible

If reversible  under what conditions.

e). What are the side reactions that may

result in unwanted products.			        Not known

f). Any isomer formation at what temperature. 	No

g). Any polymerization. If so any

inhibitors that can be tried.			                  No

h).Any sensitive groups in the reactants		No

that may need  to be protected during the reaction.

2.  Input factors:

| Sno | Factors/variables | Units | Presently used levels | State solid M.P, liquid B.P, gas | Any other information |
| --- | --- | --- | --- | --- | --- |
| 1 | Reactant 1 (cl compound) | mol | 1 | Solid |  |
| 2 | Reactant 2 (BA) | moleqv | 1.2 | solid |  |
| 3 | S1- % of water in solvent | Volume times | 50 |  |  |
| 4 | Solvent 2 (Dioxin) S2 | Volume times | 5 |  |  |
| 5 | Catalyst 1 (Pd) C | %mol | 2 | solid |  |
| 6 | Base 1 (alkali) (Pot.carb) | moleq | 3 | solid |  |
| 7 | Base 2 (alkali) (Na.carb) | moleq | 3 | solid |  |
| 8 | Temperature | Deg/c | 50-80 |  |  |

3.Output factors:

| Sno | Output factors | Units | Present level | Desired level |
| --- | --- | --- | --- | --- |
| 1 | Yield | % | 85 |  |
| 2 | Purity | % | 90% | >95% |
| 3 | Impurity specific | % | Nil |  |
| 4 | Any unreacted reactants | % | Nil |  |

4.Planning the levels of factors for experiments in DOE.

| Input factors | Present level | -2 level | -1 level | 0 level | 1 level | 2 level |
| --- | --- | --- | --- | --- | --- | --- |
| X1(S1 or S2) | 5 times |  | S1 |  | S2 |  |
| X2 (Base B1, B2) | 3moleq |  | B1 |  | B2 |  |
| X3(R2/R1) | 1.2 | 1.1 | 1.2 | 1.3 | 1.4 | 1.5 |
| X4 (C) | Moles | 0.1 | 0.7 | 1.3 | 1.9 | 2.5 |
| X5Temp | Co | 50 | 55 | 60 | 65 | 70 |

5.OBJECTIVES:

1.

2.

6.PLANNING EXPERIMENTS WITH DOE:

1.	Is it a screening? Yes screening to find out which solvent, catalyst better performing. Then we need to use M4 of screening module.

2.	Is it optimization? After.

3.	Is it a linear optimization. During optimisation

4.	Is it a non linear optimization. During optimization.

Planning experiments as per DOE ICHQ8

| Sno | Input factors/ combinations | X1solvent | X2baset | X3R2/r1 | X4Cat pd | X5 temp | Y1 yield | Y2 purity | Y3 impurity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | I | S1 | B1 | 1.2 | 0.7 | 65 |  |  |  |
| 2 | X4 | S1 | B1 | 1.2 | 1.9 | 55 |  |  |  |
| 3 | X3 | S1 | B1 | 1.4 | 0.7 | 55 |  |  |  |
| 4 | X3x4 | S1 | B1 | 1.4 | 1.9 | 65 |  |  |  |
| 5 | X2 | S1 | B2 | 1.2 | 0.7 | 55 |  |  |  |
| 6 | X2x4 | S1 | B2 | 1.2 | 1.9 | 65 |  |  |  |
| 7 | X2x3 | S1 | B2 | 1.4 | 0.7 | 65 |  |  |  |
| 8 | X2X3x4 | S1 | B2 | 1.4 | 1.9 | 55 |  |  |  |
| 9 | X1 | S2 | B1 | 1.2 | 0.7 | 55 |  |  |  |
| 10 | X1x4 | S2 | B1 | 1.2 | 1.9 | 65 |  |  |  |
| 11 | X1x3 | S2 | B1 | 1.4 | 0.7 | 65 |  |  |  |
| 12 | X1x3x4 | S2 | B1 | 1.4 | 1.9 | 55 |  |  |  |
| 13 | X1x2 | S2 | B2 | 1.2 | 0.7 | 65 |  |  |  |
| 14 | X1x2x4 | S2 | B2 | 1.2 | 1.9 | 55 |  |  |  |
| 15 | X1x2x3 | S2 | B2 | 1.4 | 0.7 | 55 |  |  |  |
| 16 | X1x2x3x4 | S2 | B2 | 1.4 | 1.9 | 65 |  |  |  |

X stands for input factors and Y stands for output factors.

7.ASSUMPTIONS/ CONDITIONS:

1.

2.

3.

8. Action Plan:

Unit No:

Head of the concerned lab:

Name of the Project:

Scientists associated with the project:

Representative of the project :

Name :